Memory management

Layer 1: The Bootloader Connection (The Memory Map)

Kernel needs to know how much memory is usable (Some RAM is not usable as reserved by motherboard for hardware - ex graphics card)

How it connects: In 16-bit bootloader stage, bootloader uses a BIOS interrupt (specifically int 0x15, EAX=0xE820) to ask the motherboard for a map of the RAM.

The Handoff: The bootloader stores this map in memory and passes a pointer (a memory address) of this map to your kernel's main() function as an argument.

Layer 2: The Physical Memory Manager (PMM)

Kernel has the map, & needs a way to hand out free memory to the system.

PMM divides all the "usable" RAM into fixed-size chunks called Page Frames. On x86 architecture, a standard frame is exactly 4096 bytes (4 Kilobytes).

How to implement: Create a Bitmap. A bitmap is just a massive array of individual bits (0s and 1s). Every single bit represents one 4KB frame of RAM. 0 means the frame is FREE. 1 means the frame is USED.

How it works: When your OS needs memory, the PMM scans the bitmap looking for a 0, flips it to a 1, and returns the physical address of that 4KB chunk.

Bitmap Allocator –

Array of 128 KB in size will be used.

Creating the Bitmap Array

Bitmap will be an array of 32-bit integers(uint32_t).

- Array Index 0: Tracks chunks 0 to 31.

- Array Index 1: Tracks chunks 32 to 63.

- ...and so on.

The Core Bitwise Operations

To manipulate individual bits inside a 32-bit integer, we have to use C's bitwise operators (|, &, ~, <<). Need three essential functions: one to mark a chunk as used, one to free it, and one to check its status.

Allocating and Freeing Memory

Functions that the rest of your kernel will actually use when it needs raw RAM.

When kernel says "Give me 4KB of RAM", the PMM scans the array, finds the first 0 bit, flips it to 1, and converts that bit number into a real physical memory address.

The Initialization Step (Connecting Layer 1 to Layer 2)

When kernel's main() function starts, the very first thing it will do is run a function like pmm_init(). This function takes the memory map provided by your bootloader (Layer 1), calculates how many total blocks of RAM exist, sets all the bits to 1 (used) by default, and then carefully sets the bits to 0 (free) only for the memory regions the bootloader told you were safe to use.

Layer 3: Virtual Memory & Paging (VMM)

kernel uses Paging to give every single program its own "fake" isolated memory space.

How to implement: x86-64 processor has a special piece of hardware the MMU (Memory Management Unit).

Create a 4-level tree of "Page Tables" in memory. Then load the address of the very top table into a special CPU register CR3

How it works: User program asks to use memory address 0x1000, the CPU looks at Page Tables.    The tables act like a dictionary, secretly translating the fake "Virtual" address (0x1000) into the real "Physical" spot (e.g., 0x8000) assigned by the PMM.

1. The 4-Level Treasure Map

In x86-64, a virtual address doesn't point to data; it is actually a set of instructions on how to navigate a 4-level maze of tables to find the real physical address.

Each table is exactly 4096 bytes (1 Page Frame). Because each entry in a table is 8 bytes long, exactly 512 entries fit into one table.

4 levels, from top to bottom:

- PML4 (Page Map Level 4) - The top-level directory.

- PDP (Page Directory Pointer)

- PD (Page Directory)

- PT (Page Table) - The final table that holds the actual physical address.

2. Defining the Tables in C

Table is just 512 entries of 64-bit integers. The lowest 12 bits of each entry are used for Flags (like read-only, user-mode, or present). The rest of the bits hold the physical address of the next table.

3. Slicing the Virtual Address

When a program asks for a virtual address like 0x400000, the CPU slices that address into pieces. Each piece tells the CPU which of the 512 entries to look at in the next table.

4. The Mapping Function

This is the core function of Virtual Memory Manager (VMM). You give it a Virtual Address and a Physical Address, and it wires up the 4 tables to connect them.

If a table in the middle of the chain doesn't exist yet, it asks the PMM (Layer 2) for a new 4KB block, clears it to zero, and links it in.

5. Activating the Tables (The CR3 Register)

Once tables are mapped, you have to tell the CPU to start using them. The CPU has a special hardware register called CR3 (Control Register 3). It only holds the physical address of your top-level PML4 table.

How they connect: We will use Layer 2 (pmm_alloc_block) to grab physical memory. Then will use Layer 3 (vmm_map_page) to give that physical memory a safe, fake virtual address for shell and user programs to use.

Layer 4: The Kernel Heap (Custom malloc and free)

PMM gives out memory in massive 4KB chunks but it can be waste if just need small bytes.

How to implement: Write a Heap Allocator. malloc() (memory allocate) and free() functions in C.

How it works: Heap Allocator asks the PMM for a few 4KB chunks. It then slices those large chunks up into smaller, custom-sized pieces based on exactly what the kernel or program requests. It uses a "Linked List" to keep track of which slices are currently being used and which are freed up to be used again.

Malloc: The Hidden Header

malloc() gives a program a slice of memory, it secretly hides a tiny "Header" right before the memory address it hands out. This header tracks the size of the slice and whether it is free or used.

Implementing malloc() (The First-Fit Algorithm)

When a program calls malloc(12), heap manager starts at the heap_head and checks every header until it finds a slice that is marked as is_free = 1 and has a size of at least 12.

If it finds one, it marks it as used. Then it returns the memory address directly after the header, so the program never accidentally overwrites the metadata!

Implementing free()

Because malloc() gives the program the address just after the header, free() simply takes that address, steps backward in memory to find the hidden header, and marks it as free.

How all 4 Layers Connect Together in main()

void kernel_main(void* memory_map_from_bootloader) {

// 1. Give the map to Layer 2 so it knows what physical RAM exists

pmm_init(memory_map_from_bootloader);

// 2. Grab a physical 4KB block to act as the top of Layer 3 Page Tables

PageTable* pml4 = pmm_alloc_block();

// 3. Map virtual addresses to physical ones safely

vmm_map_page(pml4, 0x100000, 0x100000, PAGE_PRESENT | PAGE_RW);

// 4. Activate Layer 3 so the CPU starts using Virtual Memory

vmm_activate_paging(pml4);

// 5. Ask Layer 2/3 for a chunk of memory to serve as our Heap

void* heap_memory = pmm_alloc_block();

// 6. Set up Layer 4 (The Heap Header) at the start of that block

heap_head = (HeapBlock*)heap_memory;

heap_head->size = 4096 - sizeof(HeapBlock);

heap_head->is_free = 1;

heap_head->next = NULL;

//void* text = my_malloc(100); for testing

}